# NDGS-3630 Group L2 Multicast Test

# Result

- Group L2 Multicast installation successfully and chain to two L2 Interface groups fine

- Flow to go to L2 Multicast Group successfully

# Environment

| ONOS Version | 2.2.0.rc1 |
| --- | --- |
| ONOS OpenFlow Driver | Default |
| DGS-3630 | Firmware 2.22.B007 |

winlab@root > summary node=127.0.0.1, version=2.2.0.rc1 clusterId=default-7313 nodes=1, devices=1, links=0, hosts=0, SCC(s)=1, flows=4, intents=0 winlab@root > devices id=of:000078321bdf7600, available=true, local-status=connected 1h43m ago, role=MASTER, type=SWITCH, mfr=D-Link Corporation, hw=DGS-3630-28TC H/W:A1, sw=Runtime:2.22.B007, serial=RZYG1HC000100, chassis=78321bdf7600, driver=default, channelId=192.168.20.201:1617, managementAddress=192.168.20.201, protocol=OF_13

## ONOS Apps Activation

winlab@root > apps -s -a * 6 org.onosproject.hostprovider 2.2.0.rc1 Host Location Provider * 7 org.onosproject.lldpprovider 2.2.0.rc1 LLDP Link Provider * 8 org.onosproject.optical-model 2.2.0.rc1 Optical Network Model * 9 org.onosproject.openflow-base 2.2.0.rc1 OpenFlow Base Provider * 10 org.onosproject.openflow 2.2.0.rc1 OpenFlow Provider Suite * 20 org.onosproject.drivers 2.2.0.rc1 Default Drivers * 80 org.onosproject.proxyarp 2.2.0.rc1 Proxy ARP/NDP * 104 org.onosproject.gui2 2.2.0.rc1 ONOS GUI2

# Pipeline

DGS-3630 Group L2 Multicast would go through the pipeline as following:

# Specific Format Requirment

Referring to the Spec. “M2 (2019) > OpenFlow Test > DGS-3630 > Official Document > Appendix - OpenFlow Object Details.pdf”, here grab the content and make a simple explain for this test using.

### 2.3 L2 Multicast Group Entry Type

| Field | Description |
| --- | --- |
| Group Identifier | A 32 bit unsigned integer uniquely identifying the group on the OpenFlow switch. The naming rule is shown in Table L2 Multicast Group Entry Naming Conversion. |
| Group Type | All |
| Counters | Per-group entry counters are specified. |
| Action | Support multiple action buckets. |

### L2 Multicast Group Entry Naming Conversion

| Field | Bits | Description |
| --- | --- | --- |
| Index | [15:0] | Index to this kind of groups |
| Chain ID | [27:16] | The chain ID is used to reference L2 interface group entry. The range is 1-4094. |
| Kind | [31:28] | 3 (L2 Multicast) |

- In this test, we use the id as following for L2 Multicast Group:

| Kind |  |  |  | Chain ID |  |  |  |  |  |  |  |  |  |  |  | Index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 310 |  |  |  | modify Chain ID to 110 |  |  |  |  |  |  |  |  |  |  |  | 110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 23 | 22 | 21 | 20 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 215 | 214 | 213 | 212 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |

### L2 Multicast Group Entry Bucket Actions

| Field | Description |
| --- | --- |
| Group | Must chain to an L2 Interface group entry whose Chain ID name component matches the Chain ID component of this group entry’s name. |

### L2 Multicast Group Entry Counters

| Field | Description |
| --- | --- |
| Reference Count (flow entries) | Number of flow or group entries currently referencing this group entry. |
| Duration (seconds) | Seconds since this group entry was installed |

# Test Steps

## Step1. Install L2 Interface Group

The group id as following for L2 Interface Group:

| Kind |  |  |  | Chain ID |  |  |  |  |  |  |  |  |  |  |  | Interface ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 |  |  |  | modify Chain ID to 110 |  |  |  |  |  |  |  |  |  |  |  | 110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 23 | 22 | 21 | 20 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 215 | 214 | 213 | 212 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |

- Kind=0, L2 Interface

- Chain ID=1, range is 1~4094

- Interface ID=1, for output port is 1

curl L2 Interface Group Entry:

$ curl -X POST --user onos:rocks -H "content-type:application/json" http://localhost:8181/onos/v1/groups/of:000078321bdf7600 -d @InterfaceGroup1.json $ curl -X POST --user onos:rocks -H "content-type:application/json" http://localhost:8181/onos/v1/groups/of:000078321bdf7600 -d @InterfaceGroup2.json

- InterfaceGroup1.json

{ "type":"INDIRECT", "appCookie":"0x10001", "groupId":"65537", "buckets":[ { "treatment":{ "instructions":[ { "type":"OUTPUT", "port":"1" } ] } } ] }

- InterfaceGroup2.json

{ "type":"INDIRECT", "appCookie":"0x10002", "groupId":"65538", "buckets":[ { "treatment":{ "instructions":[ { "type":"OUTPUT", "port":"2" } ] } } ] }

## Step2. Install L2 Multicast Group

| Kind |  |  |  | Chain ID |  |  |  |  |  |  |  |  |  |  |  | Index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 310 |  |  |  | modify Chain ID to 110 |  |  |  |  |  |  |  |  |  |  |  | 110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 23 | 22 | 21 | 20 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 215 | 214 | 213 | 212 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | - 20 |

curl L2 Multicast Group Entry:

$ curl -X POST --user onos:rocks -H "content-type:application/json" http://localhost:8181/onos/v1/groups/of:000078321bdf7600 -d @MulticastGroup.json

- MulticastGroup.json

{ "type":"ALL", "appCookie":"0x30010001", "groupId":"805371905", "buckets":[ { "treatment":{ "instructions":[ { "type":"GROUP", "groupId":"65537" }, { "type":"GROUP", "groupId":"65538" } ] } } ] }

## Step3. Install Flow to go to group entry

curl Flow Write-Actions:

$ curl -X POST --user onos:rocks -H "content-type:application/json" http://localhost:8181/onos/v1/flows -d @flow.json

- flow.json

{ "flows":[ { "priority":40001, "timeout":0, "isPermanent":true, "deviceId":"of:000078321bdf7600", "selector":{ "criteria":[ { "type":"ETH_TYPE", "ethType":"0x0800" }, { "type":"IPV4_DST", "ip":"192.168.202.2/24" } ] }, "treatment":{ "instructions": [], "deferred": [ { "type":"GROUP", "groupId":"805371905" } ] } } ] }

# Conclusion

- Group L2 Multicast installation successfully and chain to two L2 Interface groups fine

- Flow to go to L2 Multicast Group successfully

## ONOS GUI in Group Table

## ONOS GUI in Flow Table

## Test1 ping Test2

## Use tcpdump to monitor

## DGS-3630 CLI in Group Entries

Switch#show openflow group-desc === GroupId=65537, Hex=0x00010001 (L2 Interface, ChainId=1, Port=1) Type=Indirect, Reference count=0 Create time=2015-10-04 01:26:07 Bucket=1: Output=1 === GroupId=65538, Hex=0x00010002 (L2 Interface, ChainId=1, Port=2) Type=Indirect, Reference count=1 Create time=2015-10-04 01:26:11 Bucket=1: Output=2 === GroupId=805371905, Hex=0x30010001 (L2 Multicast, ChainId=1, Id=1) Type=All, Reference count=1 Create time=2015-10-04 01:26:18 Bucket=1: Group=65538 Total:3

## DGS-3630 CLI in Flow Entry

Switch#show openflow flows === Match Fields: ETH_TYPE=0x0800 IPV4_DST/Mask=192.168.202.0/255.255.255.0 Priority=40001 Cookie=0x00C000004FF4ACC0 Idle timeout=0, Hard timeout=0 Table=0, Packets=0, Bytes=0 Create time=2015-10-04 01:26:32 Instructions: Write-Actions: Group=805371905 === Match Fields: ETH_TYPE=0x0806 Priority=40000 Cookie=0x00010000DE52A92F Idle timeout=0, Hard timeout=0 Table=0, Packets=0, Bytes=0 Create time=2015-10-04 01:13:21 Instructions: Apply-Actions: Output=CONTROLLER(Reserved) Clear-Actions === Match Fields: ETH_TYPE=0x88CC Priority=40000 Cookie=0x00010000948A7777 Idle timeout=0, Hard timeout=0 Table=0, Packets=0, Bytes=0 Create time=2015-10-04 01:13:21 Instructions: Apply-Actions: Output=CONTROLLER(Reserved) Clear-Actions === Match Fields: ETH_TYPE=0x8942 Priority=40000 Cookie=0x00010000326F1D0A Idle timeout=0, Hard timeout=0 Table=0, Packets=0, Bytes=0 Create time=2015-10-04 01:13:21 Instructions: Apply-Actions: Output=CONTROLLER(Reserved) Clear-Actions Total:4