# Rewrite ETH_DST

# Result

- L2 Rewrite “ETH_DST” installation successfully but doesn’t pass the ping test

# Environment

| ONOS Version | 2.2.0.rc1 |
| --- | --- |
| ONOS OpenFlow Driver | Default |
| DGS-3630 | Firmware 2.22.B007 |

## ONOS Apps Activation

# Pipeline

DGS-3630 L2 Rewrite “ETH_DST” would go through the pipeline as following:

# Specific Format Requirment

Referring to the Spec. “M2 (2019) > OpenFlow Test > DGS-3630 > Official Document > Appendix - OpenFlow Object Details.pdf”, here grab the content and make a simple explain for this test using.

### 2.2 L2 Rewrite Group Entry Type

| Field | Description |
| --- | --- |
| Group Identifier | A 32 bit unsigned integer uniquely identifying the group on the OpenFlow switch. The naming rule is shown in Table L2 Rewrite Group Entry Naming Conversion. |
| Group Type | Indirect |
| Counters | Per-group entry counters are specified. |
| Action | Support single action bucket. |

### L2 Rewrite Group Entry Naming Conversion

| Field | Bits | Description |
| --- | --- | --- |
| Id | [27:0] | Index of differentiate group entries of this type. |
| Kind | [31:28] | 1 (L2 Rewrite) |

- In this test, we use the id as following for L2 Rewrite Group:

| Kind |  |  |  | ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 310 |  |  |  | 110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 23 | 22 | 21 | 20 | 227 | 226 | 225 | 224 | 223 | 222 | 221 | 220 | 219 | 218 | 217 | 216 | 215 | 214 | 213 | 212 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |

### L2 Rewrite Group Entry Bucket Actions

| Field | Description |
| --- | --- |
| Group | Must chain to a L2 Interface group entry. Required. |
| Set-Field | Set ETH_DST, ETH_SRC and VLAN_VID fields. Optional. |

### L2 Rewrite Group Entry Counters

| Field | Description |
| --- | --- |
| Reference Count (flow entries) | Number of flow or group entries currently referencing this group entry. |
| Duration (seconds) | Seconds since this group entry was installed |

# Test Steps

## Step1. Install L2 Interface Group

The group id as following for L2 Interface Group:

| Kind |  |  |  | ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 310 |  |  |  | 110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 23 | 22 | 21 | 20 | 227 | 226 | 225 | 224 | 223 | 222 | 221 | 220 | 219 | 218 | 217 | 216 | 215 | 214 | 213 | 212 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |

- Kind=0, L2 Interface

- Interface ID=2, for output port is 2

curl L2 Interface Group Entry:

$ curl –u onos:rocks –X POST –H “Content-type: application/json” -d @group2-2.json http://localhost:8181/onos/v1/groups/of:000078321bdf7600

- group2-2.json

{ "type":"INDIRECT", "appCookie":"0x1234abc3", "groupId":"65538", "buckets":[ { "treatment":{ "instructions":[ { "type":"OUTPUT", "port":"2" } ] } } ] }

## Step2. Install L2 Rewirte Group

| Kind |  |  |  | ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 310 |  |  |  | 110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 23 | 22 | 21 | 20 | 227 | 226 | 225 | 224 | 223 | 222 | 221 | 220 | 219 | 218 | 217 | 216 | 215 | 214 | 213 | 212 | 211 | 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | - 20 |

curl L2 Rewrite Group Entry:

$ curl –u onos:rocks –X POST –H “Content-type: application/json” -d @group1-1.json http://localhost:8181/onos/v1/groups/of:000078321bdf7600

- group1-1.json

{ "type":"INDIRECT", "appCookie":"0x2234abc3", "groupId":"268500994", "buckets":[ { "treatment":{ "instructions":[ { "type":"L2MODIFICATION", "subtype": "ETH_DST", "mac": "d0:94:66:5c:45:1f" }, { "type":"GROUP", "groupId":"65538", } ] } } ] }

## Step3. Install Flow to go to group entry

curl Flow Write-Actions:

$ curl -u onos:rocks -X POST -H "Content-type: application/json" -d @flowethsrc.json http://localhost:8181/onos/v1/flows

- flow.json

{ "flows":[ { "priority":100, "timeout":0, "isPermanent":true, "deviceId":"of:000078321bdf4000", "treatment":{ "instructions": [], "deferred": [ { "type":"GROUP", "groupId":"268500994" } ] }, "selector":{ "criteria":[ { "type":"IN_PORT", "port":"1" } ] } } ] }

## Step4. test1 ping test2

ping -c 5 -I enp4s0 192.168.205.2

## Step5. test2 tcpdump

ping -c 5 -I enp4s0 192.168.205.1

# Conclusion

L2 Rewrite “ETH_DST” installation successfully but doesn’t pass the ping test

## ONOS GUI in Group Table

## ONOS GUI in Flow Table

## DGS-3630 CLI in Group Entries

sudo tcpdump -i enp4s0 -e icmp

sudo tcpdump -i enp4s0 -e icmp